Routine Name:	calc_Rm

Description:	Calculates input membrane resistance (Rm) of neuron.

Usage:		calc_Rm compartment

Example:	echo { calc_Rm(/soma) }

Notes:		calc_Rm calculates the input membrane resistance using only

resistive properties (i.e., the compartment model) and

ignoring geometrical effects.

This routine uses simple branching resistive network

calculations to estimate the input resistance of the cell. It

gives exactly the same result as a compartmental simulation of

current injection would, assuming only passive properties,

since both are using the same resistive network in their

calculations.  This value differs from the true value for a

branched cylindrical structure as calculated using cable

theory, by a few (< 3) percent.

See also:	rall_calc_Rm