TOGAI INFRALOGIC INVERTED PENDULUM DEMONSTRATIONS

OVERVIEW

This diskette contains three demonstration programs which illustrate

adaptive and non-adaptive fuzzy control of an inverted pendulum.

1. PENDEMO.EXE was developed using Togai InfraLogic's Fuzzy-C

Development System.  The intermediate knowledge-base file

generated by Togai InfraLogic's Fuzzy Logic CASE tool,

TILShell, is DEMO.TIL.  The machine-generated C code, created

by Togai InfraLogic's Fuzzy-C Development System, is DEMO.C.

The fuzzy knowledge base, or FAM matrix, was developed by

observing the system and fine tuning.

2. DCLDEMO.EXE uses an adaptive vector quantizer with differential

competitive learning (DCL) to adaptively infer the FAM rules

from sample data.

3. NEWDEMO.EXE is the inverted pendulum demo with the original FAM

matrix replaced by the DCL-estimated matrix.

I. PENDEMO

In this demo, a fuzzy logic controller balances a stationary inverted

pendulum with variable mass, period, and motor strength.  The control

inputs are the angle away from vertical (Theta) and the change in

angle (dTheta).  The output is the current that drives the motor.

To run the demonstration, type "pendemo".  The demo will immediately

begin with the default settings.  Before proceeding, hit the "?" key

and page through the help menus.  This will provide an overview of

how the demonstration works.

The demonstration screen is divided into five display windows as

follows:

1. The center window shows the pendulum itself.

- The red bob at the top of the pendulum is a user-alterable

mass.	Pressing F6 increases the mass of the bob; shift-F6

decreases it.

- The green shaft separates the bob from the motor.  Pressing

the F7 key causes the length of the shaft to vary, altering

the system non-linearities considerably.

- The blue ball at the bottom of the pendulum is a motor	which

drives the shaft to the left or right.  The current supplied

to the motor is determined by the fuzzy controller.  F5 and

shift-F5 increase or decrease the strength of the motor.

The other items in the window reflect the status of these main

features.

2. The upper right window shows the FAM matrix governing the fuzzy

controller.  Each element in the matrix represents one fuzzy

associative memory or FAM rule.	For example, the center element

in the matrix reads

IF Theta is Z and dTheta is Z

THEN Current = Z.

If a rule in the knowledge base is firing, the matrix element

corresponding to the rule is highlighted in gray.

3. The lower right window consists of two main sections: the

rule list and the currently selected rule display.

You can scroll through the rule list with the up and down

arrows on your PC.  Scrolling through the list dynamically sets

the highlighted rule in the rule list to be the "current rule".

The current rule is then graphically displayed to the right of

the rule list.

Pressing the F1 key disables the current rule in the knowledge

base, preventing it from firing under any circumstances.

Pressing F1 again re-enables the rule.

4. The lower left window displays the system from a black-box

perspective.  The crisp input values, Theta and dTheta, are

shown at the top of the window, while the crisp output value,

motor current, is displayed at the bottom of the window.

The middle portion of the window shows the combined output

fuzzy set determined by all of the rules in the knowledge base

firing to different degrees.  The crisp output value (for that

cycle) is the centroid of the combined output fuzzy set.

5. The upper left window is a trace buffer showing the motor

current as it varies over time.

Each fuzzy control rule relates input fuzzy sets to an output fuzzy

set.  Hence it is a "fuzzy associative memory" (FAM).  Each antecedent

in the premise is a fuzzy association which admits a "level of

belief".  Fuzzy "membership functions" defined for each variable

determine the degree to which a "crisp," or numerical, input value

belongs to each of five fuzzy-set values: Negative Medium (NM),

Negative Small (NS), Zero (Z), Positive Small (PS), and Positive

Medium (PM).  An antecedent's level of belief is the degree to which

the crisp input value belongs to the associated fuzzy-set value.  For

example, if the crisp value of Theta belongs to the PS fuzzy set to

degree 0.8, then the fuzzy association

Theta is PS

has a 0.8 level of belief.

The output "activation" for each FAM rule is the minimum level of

belief in the premise (maximum if the antecedents are combined with

OR).  The output of each rule is a fuzzy set formed by "clipping" the

consequent fuzzy set at this activation level.  The lower right window

shows the antecedents' levels of belief, the output activation, and

the resulting clipped output fuzzy set (in red) for the current rule.

Output fuzzy sets from all of the enabled FAM rules are combined with

a logical OR to form a "combined output fuzzy set," displayed in the

lower left window.  This combined output fuzzy set is then "defuzzi-

fied", or converted to a crisp output value, by taking its centroid.

The user can test the robustness of this simple fuzzy controller by

disabling one or more FAM rules and noting the effect.	Such action

distorts or even eliminates sections of the control surface mapping

crisp inputs to crisp outputs.

II. DCLDEMO

This demo blindly estimates the pendulum's FAM matrix from sample data

vectors.  Crisp values of Theta, dTheta and Current, which appear in

the upper left window of PENDEMO, were saved in a trace file called

PEND.DAT during trial runs of the original PENDEMO.  Each data vector

is a point in the three-dimensional product space.  An adaptive vector

quantizer (AVQ) with differential  competitive learning (DCL)

adaptively clusters these data vectors to estimate the underlying

control surface which generated them.  The product space is then

partitioned into FAM cells, each one representing a single FAM rule.

On the left side of the screen, two-dimensional projections of the

data vectors appear as points in the input plane.  The color of each

point indicates which fuzzy-set value admits the highest level of

belief for that vector's crisp output value.

Cyan lines partitioning the plane indicate FAM cell boundaries.  If

the number of data vectors in a cell exceeds a threshold, then the

associated FAM rule is entered in the FAM matrix on the right side of

the screen.  Since points of more than one color often appear within

a FAM cell, we sum the consequent fuzzy sets in each cell and employ

centroid defuzzification.  The consequent fuzzy set for each FAM rule

is that output fuzzy-set value with centroid nearest the defuzzified

centroid for its cell.	After the specified number of samples are

processed, the resulting FAM matrix resembles the underlying fuzzy

control surface.

Like most neural networks, an AVQ's performance is highly data-

dependent.  Since the AVQ assumes no prior knowledge of the underlying

control surface, FAM rules that are seldom used will not appear in the

final FAM matrix.  Hence, one or more rules may need to be added to

the estimated FAM matrix to handle initial boundary conditions.

III. NEWDEMO

This demo is the same as PENDEMO, except the original FAM matrix has

been replaced by the DCL-estimated matrix.

FURTHER INFORMATION

Please direct any comments, suggestions, or questions to

Togai InfraLogic, Inc.

30 Corporate Park, Suite 107

Irvine, CA	92714

Phone: (714) 975-8522

F A X: (714) 975-8524

#############################################################################################################################