A Neural-Net Training Program Based

on Conjugate-Gradient Optimization

I.  Introduction

Neural-net classifiers have been demonstrated to be attractive

alternatives to conventional classifiers by numerous researchers

(Gorman, 1988; Waibel, 1987; Barnard, 1988; Burr, 1988).  However, there

are two	reasons why these classifiers have not gained wider acceptance:

1)  They have a reputation for being highly wasteful of

computational resources during training. In one of the first

papers on neural-net classifiers (Rumelhart, 1986), a relatively

simple two-class experiment is described that required several

thousand iterations to train, and Waibel et (Waibel, 1987)

describe a three-class problem which required more than

20,000 training iterations and several days on a supercomputer;

2)  their training has conventionally been associated with the

heuristic choice of a number of parameters; if these

parameters are chosen incorrectly, poor performance results,

yet no theoretical basis for choosing them appropriately for a

given problem exists.

In this report we describe a technique that goes some way

towards eliminating these disadvantages. To do this, we use an

optimization technique extensively studied within the

numerical-analysis literature instead of the variants of gradient

descent customarily employed with neural-net training. Use of

conventional optimization techniques for the training of neural nets

has been proposed by numerous authors (Watrous, 1987; Lapedes, 1988;

Barnard, 1988), for reasons which will become clear.  Also described

in the current report are two programs which implement the training

and testing phases of a neural-net classifier based on these ideas.

II. Conjugate-Gradient Optimization for Training Neural Nets

It is well known (Rumelhart, 1986) that the training of the

most popular neural-net classifier (known as "backpropagation" or,

less accurately, as the "multilayer perceptron") is accomplished by

the optimization of a criterion function <E>.  (The function <E>

depends on the training data according to details that will not

concern us here.)  When backpropagation was introduced originally

(Rumelhart, 1986), it was proposed that the criterion function be

optimized using gradient descent.  However, it was soon realized

that more efficient training techniques can be employed; the

"momentum" term (Rumelhart, 1986) is the most popular example of

such improvements. In this approach, not only the gradient, but

also the previous weight change is used to update the weight vector.

Various researchers have shown the usefulness of the momentum

approach; its efficacy can be understood by analogy with the quadratic

case, where it improves the minimization rate by creating a

second-order dynamic system (i.e. a system whose time behavior is

characterized by two poles in the frequency domain).  The second-order

dynamic system can be shown to have much better asymptotic convergence

properties than the first-order system that describes gradient

descent without momentum.

The momentum algorithm is characterized by two parameters (the

"learning rate" and the "momentum constant"), whose values are

critical to the success of the optimization process. Although it is

not hard to derive optimal values for these constants in the quadratic

case, no equivalent result for the more general case -- which almost

always obtains for neural-net classifiers -- exists. A large number of

heuristic algorithms that adapt these parameters according to the

training set and the status of the neural net have therefore been

proposed (see, for instance, Cater, 1987; Vogl, 1988; Jacobs, 1988).

These usually derived in ad-hoc fashion, so that neither optimality

nor robustness is ensured. In fact, in our experience most adaptive

heuristics still leave a number of problem-dependent parameters for

the user to choose, and still are rather slow to optimize <E>.

One way of overcoming these limitations is to utilize the body

of knowledge available from conventional numerical analysis.

Unconstrained optimization, which is required for the training we

study, is one of the most mature disciplines within numerical

mathematics (Gill, 1981), and supplies a wealth of applicable

knowledge.  The current opinion in the optimization community is

(Gill, 1981) that the fastest general-purpose algorithms for

unconstrained non-linear optimization are the variable- metric (also

called quasi-Newton) methods. Unfortunately, these techniques

require	O(N[2])* storage locations, where <N> is the number of

minimization variables (in our case, the number of weights). Since <N>

is often fairly large in practical neural-net applications (problems

with several thousand weights are not unusual), this storage

requirement is a major drawback.  We therefore use a

conjugate-gradient optimization algorithm (Powell, 1977).  The

conjugate-gradient algorithm is usually able to locate the minimum of

a multivariate function much faster than the gradient-descent

procedure that is customarily employed with BP.  Furthermore, its

memory usage is on the order of <N> locations. It is important to note

that the conjugate-gradient technique eliminates the choice of

critical parameters (such as the learning-rate and momentum parameters

of BP), making it much easier to use than the conventional BP training

algorithm.

* - [2] Denotes square

For a description of the principles behind conjugate-gradient

optimization, the paper by Powell (Powell, 1977) should be consulted.

III.  Programs for Training and Testing Neural Classifiers

In this section we describe the usage of a program "opt",

that implements the training phase of a neural-net classifier based

on conjugate-gradient optimization. The program was designed to allow

the experimenter to train a neural-net classifier with minimal

attention to the innards of the classifier. It is used in conjunction

with the program "optest", that tests the performance of the

classifier created with "opt".

The neural net implemented by "opt" is the "multilayer

perceptron".  It consists of <N> layers, each containing m[i] neurons

i=1,2,..,<N>.  Successive layers are completely interconnected in the

forward direction.  By choosing this structure, we have eliminated

various possibilities which have been investigated in the literature

(such as recurrent nets [Pineda, 1988], nets with certain weights

constrained to be equal [Waibel, 1987], or nets with more elaborate

connectivity [Lang, 1988]).  This was done since the structure we have

chosen is known to be computationally complete (Hornik, 1989) in the

sense that any classification boundary can be approximated to

arbitrary accuracy with this structure.  Thus, whereas other

structures might be useful for reasons such as efficiency, or for

applications going beyond conventional classification, the essential

properties of neural-net classifiers are all captured by the topology

implemented by "opt".

A.  Using "opt"

To use "opt", the user supplies two free-format text files.

The first, "opt.da<x>", contains data to specify the neural net, and

the other, "clas.da<x>", contains the training data. In both cases,

<x> is	the same integer in the range 0 - 99; it is used to label the

particular data set. The program assumes that these files are located

in the current directory (that is, the directory from which execution

is initiated).

The program is invoked with the command

opt <x> continue_flag print_flag

with <x> the integer characterizing the data set, continue_flag	a

binary digit specifying whether the net should start from random

values (continue_flag=0) or use a previously trained net as starting

point (continue_flag=1), and print_flag a binary digit indicating

whether the value of the criterion function should be reported after

every iteration (print_flag=1) or not (print_flag=0). If

continue_flag=1, the file "opt.o<x>" (see below) should exist in the

current directory. For print_flag=1, the function values are printed

to the standard output - it is often convenient to redirect these

values to a file for inspection after training has terminated.

In "opt.da<x>", the following quantities are specified

(in the order given below):

1.  <N>, the number of layers in the neural net. The input and output

layers are included in the count of <N>, and it is assumed

that <N> is not less than 3; that is, the net has at least one

hidden layer.

2.  <nit>, the maximum number of iterations of the conjugate-gradient

procedure that should be performed.

3.  <e>, the stopping criterion - if the norm of the gradient

becomes less than <e>, training is terminated; otherwise, it

continues until <nit> iterations have been completed.

4.  <p>, a seed with which the random weights are initialized.

5.  <m[sub i]> (<i=1,2,..,N>), the size of each of the <N> layers.

<m[sub 1]> should equal the number of input features, and <m[N]>

should equal the number of classes into which the samples are to

be classified. The number of output neurons equals the number

of classes since each class is represented by a separate neuron.

That is, if the <k>-th neuron in the output layer is most

active, the input is classified into class <k>. Note that

the input layer will actually contain <m[sub 1]+1> neurons,

since an "always-on" neuron is included to allow for a variable

threshold (Lippmann, 1987).  The activity of this neuron is

fixed at +1.

All these quantities are integers, except for <e>, which is a

floating-point number.

The assumption that <N is equal to or greater than 3> ensures that we

have a "true" neural classifier, not a linear classifier.  For <N=2>,

the program "opt_l" - that trains such a linear classifier - should

be used.  Except for the fact that "opt_l" assumes <N=2>, "opt" and

"opt_l" are identical. Thus, "opt_l" will not be discussed any

further.

In "clas.da<x>" we include the following data:

1.  <ntrain>, an integer equal to the number of training samples

in the file;

2.  for each of the <ntrain> training samples,

a.  an integer in the range <1,2,..,m[sub N]>, specifying

the class to which this sample belongs; and

b.  <m[sub 1]> floating-point numbers, each representing a

different feature value. To minimize local-minimum

problems, these features should be scaled so that all

or almost all values are between -1 and 1.

After training, "opt" creates a binary file "opt.o<x>" (<x> as

above) that contains the trained weights. This file is used for

further training or texting (when continue_flag=1), as is explained

below.

B.  Using "optest"

To test the performance of the trained net, "optest" is used

with the command:

optest <x y>

with <x> specifying which trained net is to be tested, and <y>

specifying the test set to be employed.  Thus, both <x> and <y> are

integers in the range 0, 1, .., 99; the trained net is contained in

the file "opt.o<x>", and the set of test samples is contained in

"clas.da<y>", the format of which is explained above.

This command produces a text file "test.<y>", containing the

percentage of test samples correctly classified and the confusion

matrix (rows correspond to the true classes and columns to the classes

assigned to samples by the neural net).

C.  Using "wread"

The program "wread" converts the binary file "opt.o<x>" into a

text file, to allow visual inspection of the weights produced by

training.  It is invoked by:

wread <x>

with <x> the integer labeling the training and specification files

used to produce the neural net.  This command produces the text file

"wread.o<x>", containing the weights in a format which is explained

below.

D.  Example

We now give examples of the files described in the previous

subsections.  Consider the exclusive-or problem, consisting of four

training samples from two classes.  To train a three-layered net with

three hidden neurons on this problem, we use the file "opt.da0" shown

in Fig. 1.

3 30 0.0000001 1989

2 3 2

Figure 1.  Contents of "opt.da0" to train on exclusive-or problem.

-------------------------------------------------------

The sample vectors used for training are contained in the file

"clas.da0" as is shown in Fig. 2.

4

1 0 0

2 0 1

2 1 0

1 1 1

Figure 2.  Contents of "clas.da0" to train on exclusive-or problem.

---------------------------------------------------

By typing:

opt 0 0 1 > e_log

we will let the conjugate-gradient optimizer run for 30 iterations

starting from random initial weights (whose values are determined by

the seed number 1989). The values of the criterion function on

successive iterations are directed to the file "e_log" (for a typical

example see Fig.  3); and the binary output file "opt.o0" contains the

resulting weights of the classifier. In Fig. 3 it can be seen how the

value of the criterion function is gradually decreased as the training

procedure iterates.

0.2910

0.2933

0.2139

0.1967

0.1693

0.2407

0.1650

0.1641

0.1622

0.1590

0.1912

0.1581

0.1579

0.1579

0.1623

0.1579

0.1574

0.1538

0.1533

0.1527

0.1509

0.1500

0.1461

0.1444

0.1373

0.1307

0.1302

0.1197

0.1131

0.1051

Figure 3.  Output produced by command "opt 0 0 1".

------------------------------------------------

For the exclusive-or problem, we wish to test on the same

samples we trained on, so "optest" is invoked with the arguments "0

0". This produces the file "test.0", shown in Fig. 4.  We see that 3

of the 4 samples are correctly classified after 30 iterations (a "1"

was called a "2"). (If we had wanted to use a different test set -

contained in "clas.da50", say - we would have given the command

"optest 0 50").

# of its.: 30; % correct= 75.00

label    1    2

----- ---- ----

1:    1    1

2:    0    2

Figure 4.  Contents of "test.0" after execution of "optest 0 0".

-----------------------------------------------------

To inspect the weights leading to this classification, we run

"wread 0".  The contents of the file "wread.o0" produced by this are

shown in Fig. 5.  As can be seen, the first several lines of"wread.o0"

describe the size of the neural net used.  (Note that the number of

input neurons is three, one more than the number of features, because

of the "always-on" neuron.  By convention, this neuron is neuron

<m>[sub 1] + 1 in the input layer, with the first <m>[sub 1] neurons

in the first layer corresponding to the <m>[sub 1] features).

Thereafter, the weights are listed, with all weights leading to a

given neuron on the same line.  For a neuron in layer <i>, the weights

from the <m>[sub i-1] neurons in layer <i>-1 are listed in order.

Thus, in our example, the second neuron in layer 2 is connected to the

first feature by a weight of 6.569, to the second feature with a

weight of -1.950, and to the always-on neuron by a weight of 0.972.

# of layers: 3

layer 1 has 3 neurons

layer 2 has 3 neurons

layer 3 has 2 neurons

Weights:

***LAYER 1 TO LAYER 2***

to neuron 1:

-9.432522 10.613865 -7.209504

To neuron 2:

6.569661 -1.950402 0.972534

To neuron 3:

1.260390 -6.663808 3.632652

***LAYER 2 TO LAYER 3***

To neuron 1:

1.109862 -0.393785 0.336293

To neuron 2:

-1.055528 0.674493 -0.387741

Value of criterion function:  0.105076

Figure 5.  Contents of "wread.o0" after execution of "wread 0"

---------------------------------------------------

Further training can be initiated by changing the number of

iterations in "opt.da0" if desired, and running "opt" with

continue_flag equal to 1.  The criterion-function values output by the

program are useful to determine whether it is sensible to continue

iterating. For the output shown in Fig. 3, further minimization is

clearly required, since the criterion function values were still

decreasing rapidly when optimization was terminated after 30

iterations. Note that the number of iterations does not accumulate

over separate runs, so the program will execute <nit> iterations after

being started regardless of the number of iterations previously

completed.

IV.  Comments on the classifier implemented

The network implemented by "opt" is fairly standard, so that

the details given (e.g. Rumelhart, 1986) are mostly applicable

here.  However, a number of points should be noted:

1.  We have implemented a training "cutoff" - instead of

using desired values of 0 and 1 for the output neurons, we

have chosen 0.3 and 0.7, respectively.  When the output

exceeds 0.7 for a desired value of 0.7, or when the output

is less than 0.3 for a desired value of 0.3, the error is

set to 0. This cutoff procedure is useful since it counteracts

overtraining. The values of	0.3 and 0.7 were chosen

heuristically; the classifier is reasonably	insensitive to

changes in these values;

2.  the initial weight choices are determined by two contradictory

factors: to break the symmetry between the various hidden neurons,

the weights should be large, but to place the initial decision

boundaries in reasonable places (to	minimize local-minimum

problems) the weights should not be too large.

"Large" and "small" are determined by the scale of the input

features, since the output of a given hidden neuron depends

on the product of the weight and feature values. In the problems

we have studied, we have scaled the inputs in the range [0,1]

or [-1,1]. With the random weights to a node with <n_to>

incoming connections scaled to be uniformly distributed between

-3/<sqrt of [n_to]> and 3/<sqrt of [n_to]>, satisfactory

performance was obtained.  (Thus, the sum of the weights leading

to a particular node is a random variable with zero mean and a

variance of 3, independent of the number of connections leading

to it.)  This scaling is used in "opt";

3.  as described by Powell (Powell, 1977), the conjugate-gradient

algorithm (with restarts) depends on a number of parameters

- e.g. line-search accuracy, the criterion for restarting, etc.

Reasonable values for these parameters were	determined by

comparing various implementations; in any case, the algorithm

is fairly insensitive to their exact values.

------------------------------------------

References

(1)	R.P. Gorman and T. J. Sejnowski, "Analysis of hidden units in

a layered network trained to classify sonar signals, "Neural

Networks, Vol. 1, pp. 75-90, 1988.

(2)	A Waibel, T. Hanazawa, G. Hinton, K. Shikano and K. J. Lang,

"Phoneme Recognition Using Time-Delay Neural Networks,"

IEEE Trans. Acoust., Speech, Signal Processing, Vol. ASSP-37,

pp. 328-339, March 1989.

(3)	E. Barnard and D. Casasent, "A comparison between criterion

functions for linear classifiers, with an application to neural

nets," IEEE Trans. Syst., Man, Cybern., Submitted, 1988.

(4)	D. J. Burr, "Experiments on neural net recognition of spoken and

written text," IEEE Trans. ASSP, Vol. ASSP-36, pp. 1162-1168,

July 1988.

(5)	D. E. Rumelhart, G. E. Hinton and R. J. Williams, "Learning

internal representations by error propagation," in Parallel

Distributed Processing.  Cambridge MA:  MIT press, pp. 318-

362, ch. 8, 1986.

(6)	R. L. Watrous, "Learning algorithms for connectionist networks:

Applied gradient methods of non-linear optimization," Inter-

national Conference on Neural Networks, IEEE, San Diego, July

1987, pp. II-619 - I-627.

(7)	A. Lapedes and R. Farber, "How neural nets work,: Neural

Information Processing Systems, D. Z. Anderson, ed., AIP,

Denver, CO, 1988, pp. 442-456.

(8)	J. P. Cater, "Successfully using peak learning rates of 10 (and

greater) in Backpropagation networks with the heuristic learning

algorithm," International Conference on Neural Networks, IEEE,

San Diego, July 1987, pp. II-645 - II-651.

(9)	T. P. Vogl, J. K. Mangis, A. K. Zigler, W. T. Zink and D. L.

Alkon, "Accelerating the convergence of the back-propagation

method," Biological Cybernetics, Vol. 59, pp. 257-264, Sept. 1988.

(10)	R. A. Jacobs, "Increased rates of convergence through learning

rate adaptation," Neural Networks, Vol. 1, pp. 295-308, 1988.

(11)	P. E. Gill, W. Murray and M. H. Wright, Practical Optimization.

London:  Academic Press, 1981.

(12)	M. J. D. Powell, "Restart Procedures for the Conjugate Gradient

Method," Mathematical Programming, Vol. 12, pp. 241-254,

April 1977.

(13)	E. Barnard, "Optimization for training neural nets," IEEE Trans.

on Pattern Analysis and Machine Intelligence, Submitted, March 1989.

(14)	F. J. Pineda, "Dynamics and architecture for neural computation,"

J. of Complexity, Vol. 4, pp. 216-245, Sept. 1988.

(15)	K. J. Lang and G. E. Hinton, "The development of the time-delay

network architecture for speech recognition", Tech. report CMU-

DS-88-152, Department of Computer Science, Carnegie-Mellon

University 1988.

(16)	K. Hornik, M. Stinchcombe and H. White, "Multilayer feed-forward

networks are universal approximators," Discussion Paper 88-45,

UCSD Department of Economics, Feb. 1989.

(17)	R. P. Lippmann, "An introduction to computing with neural nets,"

IEEE ASSP Magazine, Vol. 2, pp. 4-22, April 1987.